# Experiment 5 — Clock Generation (60ns, 25% Duty) and 1-to-4 De-Multiplexer (Switch-Level)

Author(s): 23EC30064, 23EC10030

Date: March–April 2026

# 1. Abstract

This report documents two designs: (A) a clock generator producing a 60 ns period with a 25% duty cycle, and (B) a 1-to-4 de-multiplexer built hierarchically from switch-level 1-to-2 de-mux modules. We implement both in Verilog, verify correct timing and functional behavior through simulation with Icarus Verilog, and present terminal traces demonstrating the expected waveforms and de-mux routing. Two distinct discussions analyze implementation trade-offs and practical considerations.

# 2. Objectives

Implement a clock generator with:

Period T = 60 ns

Duty cycle = 25% (Thigh = 15 ns, Tlow = 45 ns)

Initialization to 0 at t = 0

Design a 1-to-4 de-multiplexer using switch-level modeling:

Build a 1-to-2 de-mux with switch-level primitives

Compose two stages hierarchically to realize 1-to-4 behavior

Develop testbenches that exercise representative scenarios and capture results for validation.

# 3. Theory and Design Rationale

## 3.1 Clock Generation with Specified Duty Cycle

A periodic digital clock of period T and duty cycle D% has Thigh = D% × T and Tlow = (1 − D%) × T. For T = 60 ns and D = 25%, Thigh = 15 ns and Tlow = 45 ns. In a testbench environment, we can model this using delays in an always block while ensuring an initial known state (clk = 0).

## 3.2 Switch-Level De-Multiplexer

Switch-level modeling uses transistor primitives (e.g., nmos/pmos) to express connectivity rather than Boolean functions. A 1-to-2 de-mux passes the input to one of two outputs under control of a select signal; cascading stages with a 2-bit select expands to a 1-to-4 de-mux. This modeling illustrates low-level behavior, including high-impedance (‘z’) when an output is not actively driven.

# 4. Implementation Details

## 4.1 Part A — Clock Generation

File: Verilong/nig/verilong/exp5/clock_gen/clock_gen_tb.v

`timescale 1ns/1ps

module clock_gen_tb;
    reg clk;

    initial begin
        clk = 0; // Initialized to 0 at t=0
        $dumpfile("dump.vcd");
        $dumpvars(0, clock_gen_tb);
        #180 $finish; // Run for 3 cycles
    end

    always begin
        #45 clk = 1;  // Low for 45ns
        #15 clk = 0;  // High for 15ns
    end
endmodule

## 4.2 Part B — 1-to-4 De-MUX (Switch-Level)

File: Verilong/nig/verilong/exp5/demux_switch/demux1to4.v

// 1-to-2 De-MUX using Switch Level Description
module demux1to2 (
    input  wire in,
    input  wire sel,
    output wire y0,
    output wire y1
);
    wire sel_n;
    not (sel_n, sel);

    // nmos(output, input, control)
    nmos n1 (y0, in, sel_n);
    nmos n2 (y1, in, sel);
endmodule

// 1-to-4 De-MUX hierarchical design
module demux1to4 (
    input  wire in,
    input  wire [1:0] sel,
    output wire [3:0] y
);
    wire w0, w1;
    demux1to2 d_main (in, sel[1], w0, w1);
    demux1to2 d_sub0 (w0, sel[0], y[0], y[1]);
    demux1to2 d_sub1 (w1, sel[0], y[2], y[3]);
endmodule

Testbench: Verilong/nig/verilong/exp5/demux_switch/testbench.v

`timescale 1ns/1ps

module testbench;
    reg in;
    reg [1:0] sel;
    wire [3:0] y;

    demux1to4 uut (.in(in), .sel(sel), .y(y));

    initial begin
        $dumpfile("dump.vcd");
        $dumpvars(0, testbench);
        $display("Time\tIn\tSel\tY (3-2-1-0)");
        $monitor("%t\t%b\t%b\t%b", $time, in, sel, y);

        in = 1;
        #10 sel = 2'b00; #10 sel = 2'b01; #10 sel = 2'b10; #10 sel = 2'b11;
        in = 0;
        #10 sel = 2'b00; #10 sel = 2'b01; #10 sel = 2'b10; #10 sel = 2'b11;
        $finish;
    end
endmodule

# 5. Verification and Results

## 5.1 Clock Generation

The simulation trace shows the clock toggling with the desired timing: a low phase of 45 ns followed by a high phase of 15 ns, repeating with a 60 ns period. Example terminal output:

Time    Clock Value
       0        0
   45000        1
   60000        0
  105000        1
  120000        0
  165000        1
  180000        0

## 5.2 De-MUX (Switch-Level)

Simulation indicates that only the selected output reflects the input; others remain high-impedance (‘z’), consistent with pass-transistor behavior:

Time    In      Sel     Y (3-2-1-0)
       0    1       00      zzz1
   10000    1       01      zz1z
   20000    1       10      z1zz
   30000    1       11      1zzz
   40000    0       00      zzz0
   50000    0       01      zz0z
   60000    0       10      z0zz
   70000    0       11      0zzz

# 6. Discussion 1 — Design and Implementation Considerations

Duty-cycle realization: Implementing a 25% duty over a 60 ns period with explicit #delays ensures exact Thigh/Tlow in simulation; initializing clk=0 prevents transient high at t=0 and makes waveforms unambiguous.

Switch-level clarity: Using `nmos` primitives models conduction paths and high‑Z explicitly, aligning simulated outputs with physical pass behavior (selected output driven, others left floating).

Hierarchical composition: Building 1-to-4 de-mux from 1-to-2 blocks promotes reuse, readability, and testability; each stage isolates concerns (MSB split, then LSB routing).

Observability and dumps: `$dumpvars` at the top-level testbench and structured `$monitor` output provide quick validation without extensive waveform inspection.

# 7. Discussion 2 — Practicalities, Limitations, and Extensions

Real-silicon caveats: Pure nmos pass gating degrades logic-high levels; a CMOS transmission-gate or buffered stage may be needed to restore logic levels and meet noise margins.

High-impedance handling: Downstream logic must tolerate ‘z’ on unselected outputs; synthesisable RTL often replaces ‘z’ with defined 0/1 and uses enable signals for safer interfacing.

Timing robustness: The clock TB uses ideal delays; in synthesised designs, a dedicated PLL/clocking resource is preferred. For the de-mux, registering outputs can improve timing closure in larger systems.

Scalability and parameterization: The de-mux can be generalized (1-to-2^N) via generate blocks; adding assertions in the testbench (e.g., only one output equals input) improves coverage and catches wiring errors.

# 8. Conclusion

We implemented and verified a 60 ns, 25% duty-cycle clock generator and a hierarchical 1-to-4 de-multiplexer in switch-level Verilog. Simulation results validate the timing and routing behavior. The discussions outlined key design choices, physical considerations, and natural paths to parameterize and harden the designs for broader use.

# 9. Tools and Environment

HDL and Simulation: Verilog (Icarus Verilog v13.x), VVP runtime.

Waveform Viewer: GTKWave.

Host: Linux (kernel 4.18).

# 10. References

Standard digital design texts on timing, duty-cycle generation, and pass-transistor logic.

Icarus Verilog and GTKWave documentation.

Appendix — File Map

Clock TB: Verilong/nig/verilong/exp5/clock_gen/clock_gen_tb.v

De-MUX RTL: Verilong/nig/verilong/exp5/demux_switch/demux1to4.v

De-MUX TB: Verilong/nig/verilong/exp5/demux_switch/testbench.v